Advanced Computer Networks – PCS223

Assignment 4

Name – Writick Parui

Subgroup – CS4

Roll No. – 8025320111

Q.1 Create a pool on the DHCP Server router named 'xyz' and

follow all the requirements.

• Client should use DNS server IP address: 1.2.3.4

• Client should use network 192.168.10.0/24

• Client should renew IP address after every 3 hours

• Router should not respond to BOOTP

• Client should obtain IP from DHCP server

Router Configuration:

enable

configure terminal

ip dhcp pool xyz

network 192.168.10.0 255.255.255.0

dns-server 1.2.3.4

lease 0 3

no ip bootp server

exit

Interface Configuration:

interface fa0/0

ip address 192.168.10.1 255.255.255.0

no shutdown

Client Configuration:

Set PC IP configuration to DHCP mode.

Verification:

• ipconfig ® Should receive IP in range 192.168.10.x

• ping 192.168.10.1 ® Successful reply

Conclusion:

Successfully configured DHCP server on router with pool name 'xyz'. Clients dynamically received

IP address, DNS, and gateway settings. BOOTP service was disabled and connectivity was

verified.